# Voice-Controlled Robot Using Mobile App

Arduino + HC-05 Bluetooth + L298N Motor Driver

# Arduino Code

#include <SoftwareSerial.h>

SoftwareSerial bt(10, 11); // RX, TX

int IN1 = 5;
int IN2 = 6;
int IN3 = 7;
int IN4 = 8;

void setup() {
  bt.begin(9600);

  pinMode(IN1, OUTPUT);
  pinMode(IN2, OUTPUT);
  pinMode(IN3, OUTPUT);
  pinMode(IN4, OUTPUT);

  stopRobot();
}

void loop() {
  if (bt.available()) {
    String cmd = bt.readString();
    cmd.toLowerCase();

    if (cmd.indexOf("forward") != -1 || cmd.indexOf("front") != -1) {
      forward();
    }
    else if (cmd.indexOf("back") != -1) {
      backward();
    }
    else if (cmd.indexOf("left") != -1) {
      left();
    }
    else if (cmd.indexOf("right") != -1) {
      right();
    }
    else if (cmd.indexOf("stop") != -1) {
      stopRobot();
    }
  }
}

void forward() {
  digitalWrite(IN1, HIGH);
  digitalWrite(IN2, LOW);
  digitalWrite(IN3, HIGH);
  digitalWrite(IN4, LOW);
}

void backward() {
  digitalWrite(IN1, LOW);
  digitalWrite(IN2, HIGH);
  digitalWrite(IN3, LOW);
  digitalWrite(IN4, HIGH);
}

void left() {
  digitalWrite(IN1, LOW);
  digitalWrite(IN2, HIGH);
  digitalWrite(IN3, HIGH);
  digitalWrite(IN4, LOW);
}

void right() {
  digitalWrite(IN1, HIGH);
  digitalWrite(IN2, LOW);
  digitalWrite(IN3, LOW);
  digitalWrite(IN4, HIGH);
}

void stopRobot() {
  digitalWrite(IN1, LOW);
  digitalWrite(IN2, LOW);
  digitalWrite(IN3, LOW);
  digitalWrite(IN4, LOW);
}

# Command Summary

| Mobile App Command | Robot Action |
| --- | --- |
| forward / front | Move forward |
| back | Move backward |
| left | Turn left |
| right | Turn right |
| stop | Stop motors |

Wiring note: This code uses HC-05 Bluetooth on Arduino pins 10 (RX) and 11 (TX), and L298N input pins IN1–IN4 on Arduino pins 5, 6, 7, and 8. Make sure grounds are common and verify wiring before powering the motors.